RISK ASSESSMENT HST-RA-003: Visiting Company & Hire

| Reference | HST-RA-003 | Issue date | 09 May 2026 |
| --- | --- | --- | --- |
| Activity | Visiting company & hire | Review date | 09 May 2027 (or sooner on material change) |
| Location | The Hope Street Theatre, 22 Hope Street, Liverpool | Version | 1.0 |
| Assessor | Nick Rogers | Approved by | Sam Donovan |
| Methodology | HSE 5×5 risk matrix; reviewed annually or on material change | Status | Live |

## 1. Scope and methodology

This assessment covers The Hope Street Theatre's controls when hosting third-party visiting companies and hirers, including the period from initial booking, through arrival and get-in, occupation, performance and get-out. It is host-side: it sets out the risks to The Hope Street Theatre, its staff, audiences and the building arising from the activity of others, and the controls applied at the venue level. It does not cover the production-specific risks of any particular visiting production; each visiting company is required to submit its own risk assessment using the template at Appendix A, which is reviewed and approved before activity commences. Technical specifications, floor layouts, the lighting rig and the Risk Assessment Policy are available to visiting companies via the resources page at www.madeonhopestreet.com. Routine premises and audience operations are covered separately in HST-RA-001; The Hope Street Theatre's own production activity in HST-RA-002.

Risk is assessed using the HSE 5×5 matrix. Likelihood (L) and Severity (S) are scored 1–5; Risk Rating (R) = L × S. Each hazard is scored before controls (initial) and after controls (residual). Action thresholds and the matrix are set out below.

## 2. Persons at risk: legend

| S | Staff (employees and direct freelancers of HST) |
| --- | --- |
| P | Performers and cast |
| V | Visiting companies, hirers' personnel and their guests |
| C | Contractors and third-party service providers |
| A | Audience |
| M | Members of the public (foyer, deliveries, passers-by) |
| O | Others (specify in row) |

## 3. HSE 5×5 risk matrix

| Likelihood ↓ / Severity → | 1 Insignificant | 2 Minor | 3 Moderate | 4 Major | 5 Catastrophic |
| --- | --- | --- | --- | --- | --- |
| 1 Very Unlikely | 1 | 2 | 3 | 4 | 5 |
| 2 Unlikely | 2 | 4 | 6 | 8 | 10 |
| 3 Possible | 3 | 6 | 9 | 12 | 15 |
| 4 Likely | 4 | 8 | 12 | 16 | 20 |
| 5 Very Likely | 5 | 10 | 15 | 20 | 25 |

## 4. Action thresholds

| Risk score | Band | Action required |
| --- | --- | --- |
| 1–5 | Low | Acceptable. Monitor. No further action required beyond existing controls. |
| 6–15 | Medium | Tolerable. Reduce so far as is reasonably practicable. Apply additional controls where the cost is proportionate to the risk. |
| 16–25 | High | Unacceptable without further controls. Activity must not commence (or must cease) until the residual risk is reduced. |

## 5. Risk assessment

| Ref | Hazard | Persons at risk | How harm could occur | L | S | R | Control measures | L | S | R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Visiting company unfamiliarity with the venue and facilities | S, P, V, C, A | Visiting personnel unaware of evacuation routes, fire procedures, equipment idiosyncrasies or House Rules, leading to slow or unsafe evacuation, equipment damage or accidental injury. | 4 | 3 | 12 | - Technical specifications, floor layouts, lighting rig and Risk Assessment Policy made available to visiting companies via the resources page at www.madeonhopestreet.com. - Pre-arrival briefing on first call by the HST Technician covers House Rules, equipment use and the role of the HST Technician. - Evacuation routes, assembly point and fire procedures briefed orally on first call by the HST Technician. - HST Technician on duty throughout occupation; first point of contact for any safety question. | 1 | 3 | 3 |
| 2 | Absent or inadequate visiting company risk assessment (RAMS) | S, P, V, C, A | The Hope Street Theatre is unable to assess the production-specific risks of the activity, leading to unmanaged hazards, late escalation or accident. | 3 | 4 | 12 | - Submission of a completed risk assessment is a condition of hire (Hire Agreement clause 18(a)); activity does not commence until reviewed and approved. - Hirer's RAMS template (Appendix A) provided as the standard format; visiting companies are encouraged to use it. - RAMS submitted not less than 7 days before the first performance per the Hire Agreement clause 18(a); a copy is handed to the HST Technician on first call. - Received documents are reviewed by the HST Technician for production-specific matters and by the Theatre Manager for venue-level fit. - Standing declarations (Appendix A, Section A2) required regardless of whether the visiting company submits a non-template RAMS. - Right to refuse or stop activity reserved where RAMS is inadequate, where prohibited activities are present, or where actual activity diverges from the assessed activity. | 1 | 4 | 4 |
| 3 | Prohibited activities introduced by the visiting company (pyrotechnics, naked or open flame, real firearms) | S, P, V, C, A, M | Fire, burns, projectile injury, fatality. | 2 | 5 | 10 | - Standing prohibition on pyrotechnics, naked or open flame and real firearms communicated during the booking process, in the Hire Agreement (clause 17) and in Appendix A. No exceptions, including for productions that have used these effects elsewhere. - Visiting company signs declarations of compliance before activity commences (Appendix A, Section A2). - HST Technician inspects on get-in; activity stopped immediately if prohibited items are present and not removed. - Hire Agreement provides for charge-back, refund forfeiture and termination of hire in the event of breach (Hire Agreement clauses 49–50). | 1 | 5 | 5 |
| 4 | High-risk activities requiring advance notification (theatrical weapons, haze, strobe, cooking, projected substances, audience stage access, fights/stunts) | P, A, V, S | Specific injury or harm pattern depending on the activity (cuts, burns, fall, photosensitive seizure, allergic reaction, etc.) where the activity has not been notified, assessed or controlled. | 3 | 4 | 12 | - Advance notification requirements set out in the Hire Agreement and Appendix A, Section A2. - Each notified high-risk activity must be addressed in the visiting company's RAMS with proportionate, named controls. - HST Technician confirms each high-risk activity has been approved before the activity commences in rehearsal or performance. - Activity-specific controls in HST-RA-002 (in-house productions) apply equally to visiting productions and are cross-referenced where relevant. - Activity stopped if it diverges from the assessed scope or is found to involve unnotified high-risk elements. | 1 | 4 | 4 |
| 5 | Visiting company use of HST equipment | S, V, C | Equipment misuse, electrical fault, damage to equipment or fabric. | 3 | 3 | 9 | - All visiting use of HST equipment is supervised by, or undertaken in coordination with, the HST Technician. - Visiting personnel using HST equipment confirmed competent before unsupervised use; HST Technician retains discretion to restrict use. - The lighting rig is fixed and is operated only by the HST Technician; it cannot be adjusted without the Theatre Manager's permission. - Working at height by visiting personnel is generally not permitted; if exceptional access is required, this must be notified and approved in advance and supervised by the HST Technician (cross-reference HST-RA-002 Hazard 2). - Equipment condition logged at start and end of hire; damage noted, reported and charged in line with the Hire Agreement (clause 40). | 1 | 3 | 3 |
| 6 | Slips, trips and falls during get-in, get-out and rehearsal | S, V, C | Cuts, bruises, sprains or fractures from cables, set debris, tools, low-light working or congestion during shared use of the stage. | 4 | 3 | 12 | - HST Technician coordinates the get-in / get-out schedule with the visiting company; staggered access to avoid congestion. - Working light raised in auditorium and back-of-house during get-in/out; only blacked out for performance and dress. - Cables routed overhead by default; floor cables taped down and away from access routes (cross-reference HST-RA-002 Hazard 3). - Closed-toe footwear required for all personnel during get-in / get-out. - Get-out areas kept clear of debris; rolling clear-up by the visiting company under HST Technician oversight. | 2 | 3 | 6 |
| 7 | Manual handling during visiting company get-in and get-out | S, V, C | Musculoskeletal injury, sprains, hernia or crush injury from lifting, carrying or moving the visiting company's set, equipment, props and costumes. | 4 | 3 | 12 | - Manual handling of the visiting company's own set and equipment is the visiting company's responsibility; the HST Technician may assist in line with HSE manual handling guidance. - Maximum single-person lift 25 kg in line with HSE guidance; team lifts for heavier or awkward items. - HST lifting equipment is not generally available to visiting companies; the lighting rig is fixed and cannot be adjusted without the Theatre Manager's permission. Visiting companies bring their own lifting equipment for their own load where required. - Stair-carries minimised through scheduling and routing. - HST Technician may decline to assist with a lift that would breach HSE guidance or place HST staff at unreasonable risk. | 1 | 3 | 3 |
| 8 | Visiting performer welfare during rehearsal and performance | P | Injury, exhaustion, distress or safeguarding harm to visiting performers, including in fight, dance, stunt, intimacy, audience-interaction or extended-shift contexts. | 3 | 4 | 12 | - Visiting company's RAMS must address performer welfare specific to the production: choreography, fight direction, intimacy coordination, rest periods, declared injuries and conditions. - HST Technician (or visiting Stage Manager where engaged) holds show-stop authority; called without hesitation where safety is compromised. - First aid provision: HST Technicians on duty are first-aid trained; first aid kits accessible from stage and dressing rooms; AED on site (cross-reference HST-RA-001 Hazard 7). - Visiting company collects performer disclosures (injuries, conditions, photosensitivity, allergens) and accommodates them; The Hope Street Theatre does not hold this data. - Safeguarding concerns reported to the DSL, Sam Donovan. | 1 | 4 | 4 |
| 9 | Child performers in visiting productions | P (children), V, S | Physical injury, welfare or safeguarding harm, or breach of working time and licensing requirements affecting child performers in a visiting company's production. | 2 | 5 | 10 | - The visiting company is responsible for compliance with the Children (Performances and Activities) (England) Regulations 2014, including Child Performance Licences and chaperone arrangements at the prescribed ratio. - Child Performance Licences and chaperone licences provided to the Theatre Manager before the first rehearsal involving children. - The visiting company is responsible for DBS verification of all of its personnel in regulated activity; written confirmation provided to The Hope Street Theatre on request. - Designated Safeguarding Lead at The Hope Street Theatre is the Theatre Manager / Director, Sam Donovan; notified before first rehearsal. - Children not permitted in performance or back-of-house areas without their chaperone. - Standard child-performer terms incorporated in the Hire Agreement. | 1 | 5 | 5 |
| 10 | Visiting company insurance gap | V, S, A, M (and indirect financial exposure to HST) | Loss or injury arising from visiting company activity for which no insurance recovery is available, exposing The Hope Street Theatre to claims and to financial loss. | 2 | 5 | 10 | - Public Liability Insurance to a minimum of £5,000,000 and Employers' Liability Insurance to a minimum of £10,000,000 required of every visiting company (Hire Agreement clause 18(b)). - Certificates of insurance provided to the Theatre Manager not less than 1 week before the first performance per the Hire Agreement clause 18(b); checked at submission. - Hire Agreement references the insurance requirements and provides for termination in the event of breach. - Activity does not commence in the absence of valid certificates. | 1 | 5 | 5 |
| 11 | Damage to venue and heritage features | C (financial), and indirect exposure to HST | Damage to listed fabric, fixtures, equipment, finishes or fittings during visiting company occupation. | 3 | 2 | 6 | - Hire Agreement defines responsibility for damage and the basis of charge (clause 40). - No alteration to the venue or its fixtures, fittings or fabric is permitted; visiting companies cannot vary the venue. - Venue condition inspected by the HST Technician before and after the hire; condition photographs taken where appropriate. - Damage logged for charge in line with the Hire Agreement. | 1 | 2 | 2 |
| 12 | Get-out and post-occupation hazards | S, M | Tripping, cuts, struck-by or fire risks from set fragments, fixings, debris or abandoned cabling left after the visiting company has departed. | 3 | 3 | 9 | - Get-out completed immediately following the last performance (Hire Agreement clauses 8–9). - Visiting company is responsible for restoring the venue to its original state during the get-out, including a thorough de-rig and removal of all set, props, costumes and personal items (Hire Agreement clause 9). - Refuse removed by The Hope Street Theatre as part of the post-occupation clearance. - HST Technician inspects auditorium, stage, dressing rooms and back-of-house before declaring the venue clear. - Any defect or damage logged for remediation before the next public event; charged where attributable to the visiting company in line with the Hire Agreement. | 1 | 3 | 3 |

## 6. Management action plan

Open actions arising from this assessment. Items are reviewed at the monthly HST management meeting until closed.

| # | Action | Owner | Target date | Status / completion notes |
| --- | --- | --- | --- | --- |
| 1 | Confirm the resources page at www.madeonhopestreet.com is current (technical specifications, floor layouts, lighting rig and Risk Assessment Policy) and that incoming hirers are signposted to it at point of booking. | Sam Donovan / The Theatre Manager |  |  |
| 2 | Issue Appendix A (Visiting Company Risk Assessment Template) to all forward bookings; require return not less than 7 days before the first performance per the Hire Agreement clause 18(a), with a copy handed to the HST Technician on first call. | The Theatre Manager |  |  |
| 3 | Document the HST Technician's pre-arrival oral briefing (House Rules, evacuation, fire procedures, equipment, role of the HST Technician); adopt as standard. | Sam Donovan / HST Technician |  |  |
| 4 | Confirm the process for refusal or cessation of activity where prohibited or unnotified high-risk elements emerge; communicate to all HST Technicians. | Sam Donovan |  |  |

## 7. Sign-off

Signing this assessment confirms that the named individuals have read it, accept its findings and undertake to ensure the controls are implemented and maintained for the period to next review.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Assessor | Nick Rogers |  |  |
| Approver / Director | Sam Donovan |  |  |
| Theatre Manager | Jill Eason |  |  |

Document control: this risk assessment supersedes The Hope Street Theatre Risk Assessment HST/01/18 dated 23 June 2018 in respect of visiting company and hire arrangements. Routine premises and audience operations are covered separately in HST-RA-001 and in-house production activity in HST-RA-002. Appendix A (Visiting Company Risk Assessment Template) is the standard form to be completed by every visiting company before activity is approved.

APPENDIX A

Visiting Company Risk Assessment Template

This template must be completed by every visiting company or hirer of The Hope Street Theatre and submitted to the Theatre Manager not less than 7 days before the first performance, in line with the Hire Agreement clause 18(a). Completion of all sections is required before activity is approved. The Hope Street Theatre reserves the right to refuse or stop any activity that is not adequately addressed below, that diverges from the assessed scope, or that involves any prohibited activity.

## A1. Production details

| Production / event title |  |
| --- | --- |
| Visiting company name |  |
| Performance date(s) and time(s) |  |
| Get-in date and time |  |
| Get-out date and time |  |
| Director |  |
| Stage Manager (if engaged) |  |
| Production Manager / responsible person |  |
| Contact email |  |
| Contact telephone |  |

## A2. Standing declarations

Confirm each declaration. Where a declaration is N/A, indicate so. Where details are required, provide them in the right-hand column.

| # | Declaration | Confirm / details |
| --- | --- | --- |
| 1 | We confirm that no pyrotechnics, naked or open flame or real firearms will be used. These are PROHIBITED venue-wide; no exceptions. |  |
| 2 | If theatrical weapons (blunted swords, rubber knives, replica firearms, etc.) will be used, we have notified the HST Technician in advance and agreed handling, storage and on-stage procedures. Sharp-edged or pointed real blades are not permitted; swords used in performance must be blunted. |  |
| 3 | If haze effects will be used, we have notified the HST Technician in advance. (Smoke effects are not permitted.) |  |
| 4 | If strobe or intense lighting effects will be used, we have notified the HST Technician in advance and signage will be displayed at venue entry. |  |
| 5 | If cooking, hot food or hot liquids will be used, we have notified the HST Technician in advance and the activity is risk-assessed in Section A4 below. (Open-flame appliances are not permitted; see Declaration 1.) |  |
| 6 | If substances (water, foam, etc.) will be projected at the audience, we have notified the HST Technician in advance and signage will be displayed at venue entry. |  |
| 7 | If the production involves audience participation including stage access, the controls are addressed in Section A4 below. |  |
| 8 | If the production involves fights, stunts or held lifts, a fight director (or appropriately qualified equivalent) is engaged and named in Section A4 below. |  |
| 9 | Working at height by visiting personnel is generally not permitted; the lighting rig is fixed and is operated only by the HST Technician. If exceptional access is required, this has been notified and approved in advance and will be supervised by the HST Technician. |  |
| 10 | If child performers are involved, Child Performance Licences and chaperone arrangements are in place; DBS records are held by the visiting company; the DSL (Sam Donovan) has been notified. |  |
| 11 | We hold valid Public Liability Insurance to a minimum of £5,000,000 and (where applicable) Employers' Liability Insurance to a minimum of £10,000,000 (Hire Agreement clause 18(b)). Certificates are attached at Section A3. |  |

## A3. Insurance

Public Liability Insurance to a minimum of £5,000,000 and Employers' Liability Insurance to a minimum of £10,000,000 are required of every visiting company (Hire Agreement clause 18(b)). Certificates of insurance must be provided to the Theatre Manager not less than 1 week before the first performance.

| Cover | Insurer / Policy no. | Sum insured | Expiry date |
| --- | --- | --- | --- |
| Public Liability (minimum £5m) |  |  |  |
| Employers' Liability (minimum £10m) |  |  |  |

Certificate(s) attached:    ☐ Yes    ☐ No (activity will not proceed until certificates are provided)

## A4. Risk assessment

Identify each hazard arising from the production. Score using the HSE 5×5 matrix set out in the main risk assessment (see Section 3). One worked example is provided. Add as many further rows as required; copy the row format if you run out of blank rows.

| Ref | Hazard | Persons at risk | How harm could occur | L | S | R | Control measures | L | S | R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stage combat with blunted weapons (worked example) | P, A | Cuts, bruises, fractures or eye injury from contact during fight choreography; loss of control of weapons within striking distance of the audience. | 3 | 4 | 12 | - Fight choreography directed by a named fight director (or appropriately qualified equivalent); name and credentials provided to the HST Technician. - Weapons used in performance are blunted; sharp-edged or pointed real blades are not used. - Choreography rehearsed in stages from walk-through to full speed; never run cold at full speed. - Weapons checked in and out of locked storage at the start and end of each call; signed for by performers. - Notification provided to the HST Technician on first call (Standing Declaration 2); storage and on-stage procedures agreed. - Stage layout maintains safe distance from the audience; audience not within striking arc. - Performers' declared injuries and conditions accommodated. | 1 | 4 | 4 |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |

## A5. Sign-off

By signing below, the responsible person for the visiting company confirms that the information provided is accurate and complete; that the visiting company will comply with the standing declarations at Section A2 and the controls at Section A4; and that the activity will not commence until this template has been reviewed and approved by The Hope Street Theatre.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Responsible person (visiting company) |  |  |  |
| Reviewed by (HST Technician) |  |  |  |
| Approved by (Theatre Manager / Director) | Sam Donovan |  |  |